Abstract:

It seems obvious that an organic photovoltaic device is photo-excitable only when it is illuminated from its transparent electrode, say, an indium-tin-oxide (ITO) anode, so that the incident light can go through the electrode to generate photoinduced excitons in the organic layer. In the present experiment with a single-organic-layer structure of Al/CuPc/ITO, however, we find that even higher photovoltage would emerge upon illumination of the device from the Al side. Thorough investigation of the phenomenon by transient photovoltage measurements show that the extraordinarily enhanced photovoltage signals are observable for a large range of Al thickness up to 102 nm, at which the incident laser light could hardly penetrate the metal layer at all. It is thus suggested that absorption of the incident photons by the Al layer could enhance generation of photo-induced free carriers in the organic layer, giving rise to much raised photovoltage. To further confirm that this is the case, transient photovoltage measurements are also performed for single-layer devices with another structure of Au/NPB/Al/ITO, where the inserted Al layer is several nanometers thick. The results obtained support the hypothesis that, due to light absorption, holes would be generated in the Al layer and subsequently injected from Al into the organic material, resulting in further charge seperation in the organic layer by the internal built-in electric field. A detailed discussion about the mechanism will be given in the workshop presentation.